### AvanRA-600-A Latent Lift AGV

Inertial & QR Code Navigation & 600kg Load

### Product Features

- The AvanRA-600-A Latent Lift AGV body design is compatible with QR code and LSLAM navigation for handling operations. Its navigation accuracy can reach ±10mm. Secondary calibration positioning can reach ±5mm.

- It has a load capacity of 600kg. The battery capacity is 30Ah. It can operate continuously for 12 hours under comprehensive working conditions.

- The product operates stably and efficiently with a lifting stroke of 60mm. The lifting time is ≤3s. The maximum running speed without a load can reach 2.0m/s.

- Intelligent power management: It features preset multi-level power thresholds and will autonomously charge when the power drops below the threshold.

- The obstacle avoidance radar has a maximum detection distance of 4m. It can be configured for three-level detection. The horizontal obstacle avoidance angle is 180°.

- It includes left and right emergency stop buttons. It features a front and rear collision alarm design. It comes equipped with multiple safety protections.

- It utilizes an embedded charging interface design. It is compatible with both manual and automatic charging. It provides higher reliability and safety.

### Equipment Dimensions

### Specifications

| Technical Category | Item | Technical Parameters |
| --- | --- | --- |
| Basic Parameters | Model | AvanRA-600-A |
|  | Name | Latent Lift AGV |
|  | Self-weight | 140kg (±5kg) |
|  | Load weight | 600kg |
|  | L x W x H | 954(L) * 702(W) * 250(H) mm |
|  | Navigation method | QR code navigation |
|  | Lifting stroke | 60mm |
|  | Running acceleration | ≤1.0m/s² |
| Performance Parameters | Drive method | Servo motor, differential wheel drive |
|  | Positioning accuracy | ±10mm |
|  | Maximum running speed (No-load) | 2.0m/s |
|  | Running speed (Full load) | 1.0m/s |
|  | Charging method | Automatic charging, offline charging |
|  | Endurance time | ≥ 10H (20%~95%) |
| Battery Parameters | Battery type | Lithium iron phosphate (LiFePO4) 48Vdc |
|  | Battery capacity | 30Ah |
|  | Charging time | 1.5-2H |
|  | Battery life | 1500 cycles, capacity ≥80% |
| Safety Protection | Obstacle avoidance method | Front obstacle avoidance radar, front and rear collision alarms |
|  | Sound and light alarm | Supported |
|  | Emergency stop switch | Supported, one each at the front and rear |
|  | Communication method | Wi-Fi (2.4GHz & 5GHz) |
|  | Scheduling interface | Customizable, provides native protocol |